The references will be taken from the 2002 NEC.  The other documents you need are the 1996 and 1999 codebooks along with your notebook for critical information.  I will travel through the Code providing commentary and references to sections that have some impact on our industry.  In your codebook you will notice a vertical line drawn in the margin left of sections which have been revised since the last code cycle.  When you encounter these sections you may wish to review the old section to review the change. The reference articles will not be reprinted completely since you will need your codebook for reference.

Lesson 1 Questions:

1. When does the current NEC code become effective?

a. 	January 2002

b. 	Upon local or state adoption

c. 	July 2002

2. What is the purpose of the NEC Code?

a. 	The training of electrical inspectors.

b. 	As an instruction manual for electrical contractors.

c. 	The practical safeguarding of persons and property from hazards arising from the use of electricity.

d.	All the above

- Why is the code arrangement important?

a.	The chapters go from the easiest to the hardest.

b.	Chapters 1-4 refer to general wiring and 5-8 supplements and modify the others for special conditions.

c.	The chapters are arranged in the order of inspecting an electrical installation.

d.	All the above

4.  How does Article 110 Requirements for Electrical Installations relate to an          electric sign installation?

a.	Access to the electrical equipment (Signs).

b.	Approval of conductors and equipment (Signs).

c.	Require installation instructions for listed equipment (Signs).

d.	All the above.

Answers:

- b

- c

- b

- d

Lesson 2 Questions:

1. Where is the information to size the primary branch circuits?

a.	Article 210

b.	Article 225

c.	Article 600

2. What are considered suitable for grounding and bonding?

  - Listed pressure connectors

  - Sheet metal screws

  - Soldered connections

  - All the above

3. What colors are suitable for insulated bonding conductors?

a	White

b	White with green strip

c	Green

d	All the above

Answers:

- c

- a

- c

Lesson 3 Questions

- Why is Chapter 3 important to the complete installation?

  - It outlines the wiring schematics

  - It outlines the wiring methods used in all wiring installations

  - It tells us how to bend conduit

  - All the above

- How does Chapter 3 deal with conductors?

  - Specifies the minimum size of conductors

  - Refers to other articles for modification.

  - Allows secondary and primary wiring in the same space

  - All the above

- Will GTO wire be found in this section?

  - Yes

  - No

  - When provided with sleeving

Answers:

- b

- d

- b

Lesson 4 Questions:

- Where can you use 3/8” trade size Flexible Metal Conduit (FMC)?

  - In a wet location

  - For primary wiring in lengths not exceeding 6’

  - For secondary conductors over 1000 volts

  - All the above

2.  Where can Liquidtight Flexible Metal Conduit (LFMC) be used?

a.	For secondary conductors over1000 volts in ½” trade size

b.	In a wet location

c.	For primary wiring

d.	All the above

3. Where can Electrical Nonmetallic Tubing (ENT) be used?

a.	In dry location for primary wiring

b.	For secondary wiring over 1000 volts

c.	In a wet location

Answers:

- b

- d

- a

Lesson 5 Questions:

1. When an external switch is connected and placed on the raceway of a sign how many purposes can it have?

a. 	Convenience

b.	As the disconnect within site

c.	For fire department use

d.	All the above

2. How is Article 600 different from the first four Chapters?

- It explains how to engineer a sign

- It can supplement and modify other sections of the code

- It specifies how many signs can go on a building or pole

- All the above

3. When do electric signs require Listing?

- When local zoning has adopted the NEC

- When the State has adopted the NEC

- All the above

Answers:

- d

- b

- c

Lesson 6 Questions:

1. Marking and labels when do they need to be visible?

- During installation and relamping

- When the owners representatives are present

- Upon annual return trips by the zoning officials

- All the above

2. How does the wiring methods in 600.5 (C) (2) modify Chapter 3 wiring methods?

- It will allow primary and secondary conductors in the same raceway

- It will allow flood lights to be mounted to signs and connections run in the raceways

- It will allow primary and secondary wiring in our transformer boxes

- All the above

3. When does the branch or control circuit to a sign need to be lockable?

- When it is convenient for the owner to shut off

- When it is out of site of the sign

- During electrical inspection

- All the above

Answers:

  - a

  - d

  - b

Lesson 7 Questions:

1.  When is it required to bond all dead metal parts to an equipment grounding conductor?

- Only when an electrical inspection is required

- Always

- Only when Listed signs are installed on metal poles

- All the above

2. Can equipment grounding conductors be installed in plastic conduit with secondary conductors?

- Yes

- No

- When electrical inspection is required

3. Does Article 250 Grounding and Bonding apply to signs?

- Yes

- When modified and supplemented by article 600

- When mounted on metal poles

- All the above

Answers:

1. 	b

2. 	b

3.	d

Lesson 8 Questions:

1. Do all of our enclosures require to be made of metal?

- Yes

- Enclosures can be made of other materials if Listed for the purpose

- When the enclosures are less than 1000 volts

- All the above

2.  Do Field installed section letter signs require field inspection?

- When installed in conformance with their installation instructions

- Yes

- When field connections are required

- All the above

3. Can transformers be installed over a suspended ceiling?

- When securely fastened in place and direct wired

- When cord connected

- When mounted directly to the ceiling frame

- All the above

Answers:

- b

- d

- a

Lesson 9 Questions:

1.	Is secondary-circuit ground fault protection required on all transformers?

  - Yes

  - Only on transformers rated over 1000 volts

  - No

  - Only on transformers rated at 7500 volts to ground

2.	Does Part II apply to all electric signs?

- No

- Only for field installed neon signs

- Only to Listed signs

- Yes

3.	Shall wiring methods conform to Chapter 3

- Yes

- Unless modified by Article 600

- Primary wiring shall conform to Chapter 3

- All the above

4.	When using non-metallic as a raceway for GTO wire which will apply?

  - GTO wire shall be # 14 AWG only

  - Non-metallic conduit must be installed 1 ½ from a ground plane

  - GTO wire shall be installed in sleeving

  - All the above

Answers:

- c

- b

- d

- b

Lesson 10 Questions:

1.	What wiring methods can be used with GTO cable?

- Shall be spaced 1 ½” from other conductors

- Can have zero spacing in a metal conduit

- Shall be installed where they are not subjected to physical damage

- All the above

2.	How far can you run a secondary circuit (GTO) conductor in metal conduit from the transformer or power supply to the first tube electrode connection?

- Unlimited as long as it is in sleeving

- 20 feet total

- 40 feet total

- 50 feet total

3.	Do electrode receptacles require Listing?

- Only when installed in a wet location

- No

- Yes

- As required by NFPA

4.	Do metal letters illuminated with low voltage LEDS require equipment grounding?

- Yes

- No

- Only when used outdoors

Answers:

- d

- b

- c

- a